# Tips on securing the EdgeMarc against vulnerabilities, hackers and fraudulent calls

| ## Information |  |
| --- | --- |
| Article Number | 000001396 |

| Question | How do I properly secure the EdgeMarc appliance against hackers and fraudulent calls ? |
| --- | --- |
| Answer | It is recommended that you follow these steps below to secure your EdgeMarc against hackers and fradualent calls. 1- Follow the steps outlined in this article to secure your EdgeMarc by changing all the passwords and limiting access to specified management IP addresses. 2- It is recommended that you do the following, to secure the EdgeMarc's ALG (against fradualent calls) by limiting access to whom can call into or out from, the PBX or phones on the LAN side. a) From the GUI, go to the Voip -> SIP page. Enable 'Limit inbound to listed Proxies / SIP servers' and 'Limit outbound to listed Proxies / SIP servers' The EdgeMarc will only allow SIP traffic to and from the server listed in the 'SIP server address' field. If there are other Proxies that need to be allowed, add it into the 'Allowed SIP Proxies' list. b) In the earlier versions, there was a field on the VoIP-ALG page called 'Allow WAN clients'. Please DO NOT enable that. 3- Here are some other things to note: a) If you are not using the PPTP Server feature on the EdgeMarc, make sure it's disabled. If you are using it, make sure that the password is difficult to guess. b) Do not use the DID as the password for the IP Phones / SIP UA/ SIP GW. Change it to something difficult to guess. c) If you believe that your EdgeMarc was compromised before, it is highly recommended that you factory reset the EdgeMarc and start programming it from scratch. Do not restore any saved configurations. It is also advisable to change all passwords including IP phones / SIP UA and SIP GW passwords. d) You can enable 'User Management' on the device if you need session management, user account management, and password management. Refer to the User Guide for more details before enabling this feature. e) VOS 14.1 and above will force you to change your passwords if they are still the default passwords that was shipped with the unit. 4- TCP Forwarding / Tunneling over SSH It is possible that hackers could use this feature, to tunnel over an SSH session. They could then use the EdgeMarc as a proxy, to proxy traffic to third site. By doing so, hackers could mask their true source IP address. It is highly recommended that you enable the Firewall / Security and trusted management feature on the EdgeMarc (as in Step 1). Doing so would mean that only trusted hosts will be able to connect and manage the EdgeMarc. All other attempts from unknown IP addresses will be denied. VOS 13.12.2 disables TCP Forwarding over SSH (by default). If you were using this feature prior to this release, upgrading to this release will disable that feature. If you need it re-enabled, please contact Edgewater Networks TAC for further instructions. This is a recommended release for all EdgeMarcs on the 13.x code branch (prior to this version). Please refer to the R/N: EdgeMarc VOS v13.12 branch for more details. VOS 11.6.22 addresses this issue on the 11.6.x branch. Please refer to the R/N: EdgeMarc VOS v11.6.22 for more details. 5- CVE [2014-6271] & CVE [2014-7169] / Shellshock CVE-2014-6271 also known as Shellshock is a vulnerability in the shell ‘Bash’. Bash is used by many features, such as SSH, to provide a shell, but also by web servers when loading a web page (depending on how the web server is configured), and potentially many other features, e.g. DHCP. This vulnerability is exposed in SSH and HTTP, but only to authenticated users. That is, you need to have to have the credentials to log into the system in order to use the exploit. Edgewater Networks is working to patch this vulnerability on all our products. VOS 13.12.2 contains a fix to bash that addresses this issue. This is a recommended release for all EdgeMarcs on the 13.x code branch (prior to this version). Please refer to the R/N: EdgeMarc VOS v13.12 branch for more details. VOS 11.6.22 addresses this issue on the 11.6.x branch. Please refer to the R/N: EdgeMarc VOS v11.6.22 for more details. In the meantime it is recommended that you follow our guide on securing the product, found in Step 1 above. - Enable trusted hosts to ensure that only trusted IP addresses can manage the device. - Make sure you change your passwords to strong password (a mix of alpha-numeric characters and also upper case and lower case) - You can also disable HTTP and SSH in the security page so that no one can access it from the WAN, and turn it on only when needed. - You can also enable TACACS / RADIUS authentication on the EdgeMarc if you have a TACACS or RADIUS server. That would prevent anyone who doesn't have a TACACS / RADIUS account from logging into the box. 6- CVE [2014-3566] / POODLE CVE-2014-3566 also known as POODLE (Padding Oracle On Downgraded Legacy Encryption) is a man-in-the-middle (MITM) attack that can cause a client to downgrade the security of the connection from using TLSv1.x to use SSLv3. Once an SSLv3 connection has been established, the attacker could then gain access to session cookies, which may provide further access. Note that this is in effect a client (browser) vulnerability and that the attacker must have access to the network between the client and the server. This would typically affect people connecting from a public hotspot that is untrusted. Steps to mitigate the issue - Secure your EdgeMarcs by adding trusted hosts and changing all the default passwords on the box - As this is a client (browser) vulnerability issue, it is recommended that you check your browsers to make sure that they have the latest security patches or workaround to secure your browser. Here is a link where you can read more about it (https://zmap.io/sslv3/browsers.html) You can read more about this vulnerability on these sites: http://web.nvd.nist.gov/view/vuln/detail?vulnId=CVE-2014-3566 https://www.imperialviolet.org/2014/10/14/poodle.html https://zmap.io/sslv3/ VOS 13.12.3 contains a fix to bash that addresses this issue. This is a recommended release for all EdgeMarcs on the 13.x code branch (prior to this version). Please refer to the R/N: EdgeMarc VOS v13.12 branch for more details. VOS 11.6.23 contains a fix to bash that addresses this issue. This is a recommended release for all EdgeMarcs on the 11.6.x code branch (prior to this version). Please refer to the R/N: EdgeMarc VOS v11.6.23 for more details. 7- [CVE?2015?0235] / Ghost vulnerability GHOST is a vulnerability in the family of function calls gethostbyname*() (ghost = gethost) provided by the C-library glibc. Glibc is a widely used C-library that provides common functionality for C or C++ programs such as DNS resolution. gethostbyname() is used by programs to look up DNS names and resolve them into IP addresses. This vulnerability allows the caller of the function to provide input data that will overrun the provided buffer used to contain the resolution of the DNS lookup. The data put in the buffer overrun can then be used to execute malicious code on the system. This means that the attack must be executed locally by a program that does DNS resolution and calls thegethostbyname() function. Steps to mitigate the issue For the attack to happen, the hacker must first connect to the device. Therefore, it is advisable to follow the best practice recommendations by Edgewater Networks, to lock down access to the box and only allow trusted IP addresses to manage the device. VOS 11.6.23 contains a fix to bash that addresses this issue. This is a recommended release for all EdgeMarcs on the 11.6.x code branch (prior to this version). Please refer to the VOS 11.6.23 release notes for more details 8- [CVE-2001-0020] [CVE-2001-0783] TFTP Server Directory Traversal Vulnerability Some TFTP servers do not validate input. It is possible for a remote user to connect to the TFTPD, and upon connecting, request a file in the directory above the TFTP root directory using the dot-dot notation (..). Upon doing so, a remote user may traverse the entire directory structure, and potentially download any file contained within the directory tree of the drive hosting the TFTP root directory. Engineering is currently investigating the effects of this vulnerability on all Edgewater Networks products and we will update this document with the latest findings as they become available. VOS 11.6.23 contains a fix to bash that addresses this issue. This is a recommended release for all EdgeMarcs on the 11.6.x code branch (prior to this version). Please refer to the R/N: EdgeMarc VOS v11.6.23 for more details. |