# Common CLI commands in the EdgeMarc

| ## Information |  |
| --- | --- |
| Article Number | 000001414 |

| Question | What are some of the common CLI commands in the EdgeMarc? |
| --- | --- |
| Answer | Q1: What is the command to mount a 'ramdisk' ? A1: # mkdir /var/ramdisk # mount -t tmpfs tmpfs /var/ramdisk -o size=4M (if you want an 8MB ramdisk, replace size=4M with size=8M) Q2: How do I unmount a ramdisk ? A2: # cd / # umount /var/ramdisk Q3: How do I see SIP messages using tcpdump ? A3: # tcpdump -s 0 -ni eth0 port 5060 (-s 0 is to show the full packet, n is don't resolve IP to name, i is for interface) # tcpdump -s 0 -ni eth0.1 port 5060 (if the phones are on vlan 1) # tcpdump -s 0 -ni lo (to see SIP messages for the FXS and FXO port, trace the loopback interface) # tcpdump -s 0 -ni eth0 port 5060 -w /var/ramdisk/eth0.pcap (to write the output to a pcap file) Q4: How do I add a default route in the CLI? A4: route add default gw 1.2.3.4 Q5: I have multiple vlans on the lan side of my EdgeMarc and I want to block access between them but allow them to all go out to the internet. A5: Add the following rule to the User Commands page on the GUI. iptables -D FORWARD -i eth0.+ -o eth0.+ -j ACCEPT Q6: How do I look at the interface link stats, of a 5300 LF2, in the CLI? A6: To see the link stats (same as the Set Link page in the GUI), you can run the following commmands: # ethtool eth0 # ethtool eth1 Q7: Is there a way to reset the ifconfig stats from the CLI, without disrupting service? A7: The following command reset the interface stats # reset_stats –i eth[interface value] Here is an example: # ifconfig eth1 eth1 Link encap:Ethernet HWaddr 00:90:FB:34:7B:00 inet addr:12.18.13.132 Bcast:12.18.13.143 Mask:255.255.255.240 inet6 addr: fe80::290:fbff:fe34:7b98/64 Scope:Link UP BROADCAST RUNNING MULTICAST MTU:1500 Metric:1 RX packets:48392 errors:0 dropped:0 overruns:0 frame:0 TX packets:49561 errors:0 dropped:0 overruns:0 carrier:0 collisions:0 txqueuelen:1000 Memory:feae0000-feb00000 # reset_stats -i eth1 Reset of stats successful for eth1 # ifconfig eth1 eth1 Link encap:Ethernet HWaddr 00:90:FB:34:7B:00 inet addr:12.18.13.132 Bcast:12.18.13.143 Mask:255.255.255.240 inet6 addr: fe80::290:fbff:fe34:7b98/64 Scope:Link UP BROADCAST RUNNING MULTICAST MTU:1500 Metric:1 RX packets:67 errors:0 dropped:0 overruns:0 frame:0 TX packets:63 errors:0 dropped:0 overruns:0 carrier:0 collisions:0 txqueuelen:1000 Memory:feae0000-feb00000. |

| Last Modified Date | 2/29/2016 5:29 PM |
| --- | --- |