# FAQ: EdgeMarc

| ## Information |  |
| --- | --- |
| Article Number | 000001439 |

| Question | Here is a list of generic questions and answers. All effort is done to keep it up to date but please check with Edgewater Networks TAC (support@edgewaternetworks.com) for the latest. |
| --- | --- |
| Answer | Q1. Does the EdgeMarc support T38 fax? A1. Yes, the EdgeMarc supports T38 on the FXS ports but not the PRI. Q2. Does the EdgeMarc support RFC 2833 DTMF events (for PRI PBXs) ? A2. RFC 2833 is supported on the EdgeMarcs (with PRI connections) which contain a DSP. To see if your EdgeMarc is one of those, refer to How do you tell if an EdgeMarc has a DSP or not? Q3. Which model and version of EdgeMarc supports BGP ? A3. Today , the only models that support BGP are the 4500 series and the 4600 series, running VOS 9.12.x or later. Q4. Does the EdgeMarc 200 series support Wan Link Redundacy on the ADSL interface? A4. No it doesn't. Only the EdgeMarc 250 supports WLR on the ADSL interface. Q5. How can I troubleshoot Proxy ARP issues? A5. Please refer to the article FAQ: Proxy Arp. Q6. How many IPSec VPNs are supported on the 4200, 4300 and 4500 platforms? A6. 12 Q7. Is there a list of CLI commands that I can run on the EdgeMarc? A7. Please refer to the article Common CLI commands in the EdgeMarc Q8. Does the EM 455x and 4600 series support High Avaliability? A8. Yes, the EM 455x and 4600 series supports High Availabity from VOS 11.6.0 onwards. Q9. Does HA support a pair of different HW models? A9. HA is only available for hardware systems of the same type (e.g. an EM 5300LF2 must be paired with another EM 5300LF2) and same VOS version. Q10. Is there a recommended method to upgrade the VOS on a HA pair? A10. Any software upgrade to an HA pair will require a maintenance window as you cannot currently upgrade the standby, perform an activity switch and then upgrade the newly inactive system. Calls may not persist during the switchovers. Q11. Which platforms support IPv6 ? A11. The following platforms support IPv6 with VOS 11.x and above. EdgeMarc 455x, 457x, 46xx, 5300 LF2 Q12. Is it possible to do a UDP trace route from a PC behind the EdgeMarc? A12. Add the following iptables rule in the "User Command" Page of the EdgeMarc GUI; iptables -I POSTROUTING -t mangle -p icmp --icmp-type port-unreachable -s IP_ADDRESS -j ACCEPT Replace "IP_ADDRESS" with the network or host that you are trying to UDP trace route to. Example: 12.12.12.0/24 or 12.12.12.1 (Network or Host) Q13. When I hardcode a link on the 'Set Link' page to 100-Full Duplex, why does the Link Partner only reports 100-HD? A13. When you hardcode a link to 100-FD, the link partner should be ignored. The link is already forced to negotiate at 100-FD. The Link Partner is only applicable when the link is set to Auto-Negotiate. Q14. Does the T1 PRI in the EdgeMarc support the codec, A-Law ? A14. The T1 PRI only supports U law (which is common in North America) A-Law is only support on the E1 PRI (which is common in Europe and Asia) Q15. We are trying to restrict outbound port 25 to only the Exchange server and block any other IP address from sending via port 25 outbound. How can we do this? A15. Enter the following iptables commands into the User Commands page . Modify the rule accordingly to suit your needs. iptables -I fwd_chain -p tcp --dport 25 -j DROP iptables -I fwd_chain -i eth0 -p tcp -s 192.168.1.5 --dport 25 -j ACCEPT where eth0 in your lan interface (if you have vlan, you have to modify accordingly) 192.168.1.5 is your exchange server Q16. How many WAN VLANs does the EdgeMarc support? A16. Up to 16. |

| Last Modified Date | 6/14/2016 7:30 PM |
| --- | --- |