# How do you tell if an EdgeMarc has an integrated hardware DSP or not?

| ## Information |  |
| --- | --- |
| Article Number | 000001469 |

| Question | What other models have an intergrated Hardware DSP? |
| --- | --- |
| Answer | The following models have a hardware DSP integrated in the platform EdgeMarc 4508PoE EdgeMarc 4608PoE EdgeMarc 4601 EdgeMarc 4806 EdgeMarc 4808 Other ways of identifying the presence of a DSP Assembly Number / Model number Here is the difference: 120-4508T4WNPOE24-02-A : "N" stands for "Non-DSP" 120-4508T4WDPOE24-02-A : "D" stands for "DSP" Same goes for 12port versions and 4600POE series. From the CLI, you can run the following command cat /var/ewn_model: In 8.8.3.x on a 4508PoE: The system has a DSP: MODEL_HAS_DSP=1 No DSP is present: MODEL_HAS_DSP=0 In 9.4.x and 9.7.x on a 4508PoE: The system has a DSP: MODEL_HAS_DSP=on The variable is not present if the system does not have a DSP. In 9.4.x and 9.7.x on a 4608PoE: The system has a DSP: MODEL_HAS_DSP=on No DSP is present: MODEL_HAS_DSP=off. |

| Last Modified Date | 8/8/2016 6:14 PM |
| --- | --- |