# How to recover an EdgeMarc 4700 series router

| ## Information |  |
| --- | --- |
| Article Number | 000001537 |

| Goal | How to recovery and EdgeMarc 4700 series router? How do you recover an EdgeMarc 4700 that boots up in "Recovery" mode?. |
| --- | --- |
| Solution |  | By default, the 4700 will load the recovery after 3 consecutive failed attempts to load the normal VOS image. - Power the unit on - Allow u-boot to begin loading Linux (Uncompressing Kernel Image ...) - Power off the system - Repeat 3 times - Once the system has failed to boot, it will display messages indicating the number of failed boot attempts. - After 3 or more consecutive failed boot attempts, the system will boot the recovery image. Note: the number of failed boot attempts is configurable. The default for failed boot attempts is 3. - The prompt will change to {model}_Recovery. - The version string will contain "Recovery" #### Restoring VOS while running in the recovery image The command to update VOS while running in the recovery image is the same command that used when running in normal VOS. A successful netflash will allow normal VOS to run after the next reboot. netflash will reset the failed boot counter back to 0 once it has completed the upgrade (before it reboots). netflash -fkiH 192.168.1.200 pub/e_4700/<filename>.bin where 192.168.1.200 is your ftp server <filename>.bin is the name of the VOS file on your FTP server.. |

| Last Modified Date | 2/29/2016 5:30 PM |
| --- | --- |