HMRs

HMR stands for "Header Manipulation Rules" and is something you would want to create in an EdgeMarc for achieving interoperability between a pbx and a provider for certain calls to work. You see some pbxs are limited and different in how they send their headers, it could be these headers are not acceptable to the provider when being sent out so this could result in the provider rejecting the call. The solution to this is actually having the EdgeMarc change these headers on the pbxs behalf and "regenerating" the header so that when the provider receives it, it accepts the call. The EdgeMarc does this on the B2BUA page and allows the change of specific headers like From, To, etc. Not all pbxs need this, but some may depending on how different their outgoing headers are. The following shows how these hmrs are added. First of all when you click the option list of headers in an EM you will then see a specific list of headers.

If you see the header that you want to change here, then perfect. If not you must select "Other" and enter the name of the header in the input box exactly as you want it to appear in the SIP message.

In above example, "Diversion" was entered.

When entering a value, the first thing to know is when you are entering a value for a header, the header value is entered as a series of segments where each segment is separated by a + sign.

Segments are one of 3:

1.) Literal: These are literals that are "literally" copied as is character for character and inserted into the header. They are separated by single quotes like this 'sip:' and anything you write will appear in the header. So if you write 'Excellent' then this will appear in the header. Just copies from value field and pastes into header.

2.) Variable: These are system predefined values. They are called variables because they are "extracted" from the "received" SIP message, then pasted into that specific field. They are always preceded with $ like$from.uri.user and separated by +.. This example "extracts" the user name portion of the URI from the received message, then pastes it into that field.  This is important list of variables:

3.) Function: Expressions that perform a specific operation with variables. For example you can "replace" a value in a received message to something you want instead, like replacing a +1  with nothing when leaving the EM. This is done like this replace($to.uri.user, '+1', '') , this example replaces the +1 in the to uri portion with nothing, effectively deleting it. This example is useful if you want to modify E.164. The main function is replace and here is its format.

Another function is a substring function. This function essentially doesn't 'replace' a specific digit(s) but rather 'removes' a digit or digits from a 'specific' position within the number.

To summarize, the 'start_index' are the first number of digits starting from the left and the 'len' which is 0 here means remove entire string. Its very important that you know when to use the substr function and the replacefunction. The replace function is meant to replace not to delete. You can use the replace function to delete like with this example that deletes '+1' and replaces it with 'nothing':

replace($to.uri.user, '+1', '') , however this only works in this example because +1 only exists at the beginning of the number. So if you dialed +1408-555-1155 then it would change it to 408-555-1155 and would work. However lets say you changed it to replace($to.uri.user, '1', '') , this says replace every 1 digit. which create problems because if you dialed 1408-555-1155 then it would change it to 408-555-55 and delete 'all' 1 digits. You can see the difference!

EXAMPLE DEMONSTRATIONS:

NOTE: The following require <> around their value: (black means standard & always should be there).

FORMAT:

From: Requires <>

from name <sip:DID@IP-Address:Port>

To: Requires <>

to name <sip:DID@IP-Address:Port>

Contact: Requires <>

contact name <sip:DID@IP-Address:Port>

Request-URI: <> NOT Required

sip:DID@IP-Address:Port

1.) Literals-

Lets assume for now that you want to change a header that is already in the list above. Let us change the To header with a Literal. I am going to select 'To' then enter the following value:

Now lets see what happens when we submit this and a call is made.

Very interesting indeed. It copied it exactly as we entered it between single quotations, hence a literal. Lets try it with From:

That's basically how you enter a literal & as you see it just copies & pastes.

2.) Variables-

Lets now do an example for variables. Lets assume we want to change the Request-URI. This is how it will look before we make a change:

Now lets say I want to change the domain above into a different domain yet keep other fields as is. This is what I would do:

And after submitting this is what it shows:

As you can see it only replaced the domain as suspected. Lets analyse what happened:

3.) Functions-

Lets check out an example for functions. Lets do a replace function together. Let us replace the outgoing DID number area code and change it from 678 to 408. This is what we do:

Lets see now what the EM does when it receives a dialed number from the pbx starting with 678.

As shown it replaced the area code to 408 in INVITE going to the provider. Cool!

Lets try an example with substr function. Lets delete the first character to the left of a number in request-uri, lets strip the 1 in 16782344113. This is what we add:

This is what happens when call is made:

That's great, it deleted the 1 of the INVITE going to the provider. Lets say though we wanted to delete the first 4 digits from the left, this is what we would enter:

Lets see what happens when we make the same call:

As shown the first four digits from left were deleted.

This concludes the HMR tutorial. I hope you had fun.