# Lab 1.4: Wireshark Familiarization

# Lab Overview

Capture LAN traffic using the Wireshark protocol analyzer.

# Duration

- 10 minutes

# Prerequisites

None

# Objectives

After successful completion of this lab, students will be able to:

- Start & run wireshark

- Apply Display filters to captured data packets

- Understand the basic physical capture configuration options with Wireshark

- Understand the pcap file format and save / share captured data.

# Lab Equipment – Per Group Requirement

- Computer running Wireshark

# Shared Equipment – Demos & Labs

- Internet Access

- Lab network

- Internet connectivity

# Supporting Documents

https://wireshark.org/docs

# Procedure

- Start Wireshark. Select the active interface.

- Follow classroom demonstration steps to capture data packets

- Apply Display filter to isolate the following types of packets:

  - -arp

  - -bootp

  - -http

  - -udp

  - -ip.addr==192.168.1.1

# Questions

- What is the current release of Wireshark?

- What is a pcap file?

- Can you enter a display filter using CAPITAL letters?

- What interfaces are available on your computer to capture data packets?

- Can you “spy” on other people in a coffee shop using Wireshark with your WiFi interface?

- What is Ethernet “port mirroring” and how is it used with a computer running Wireshark?

- How could you use a “Hub” to help capture data packets sent/received at an IP Phone?