# Lab 3.4: Setting up VLAN

# Lab Overview

Setup and manage EdgeMarc LAN & WAN VLAN Options. VLANs are commonly deployed in enterprise networks to isolate Ethernet frames containing real time voice and video from other “data” traffic. Setting up a Voice specific VLAN has many advantages including:

- Ensuring better match between available network bandwidth and Voice traffic requirements

- Separate Layer 3 DHCP, LDAP, and other support services

- Security of voice traffic transmission

# Duration

- 30 minutes

# Prerequisites

None

# Objectives

- After successful completion of this lab, students will be able to:

  - Understand EdgeMarc VLAN configuration options

  - Implement LAN VLAN configurations

  - Implement WAN VLAN configurations

# Lab Equipment – Per Group Requirement

- Computer running Wireshark

- EM 4700

# Shared Equipment – Demos & Labs

- Lab network

# Supporting Documents

EM User Guides

# Procedure

Part 1: Enabling LAN side VLAN support.

Determine a VLAN deployment strategy for all VoIP devices. Create a VLAN ID = 500 for VoIP devices, determine if VLAN will be implemented on a port by port basis using “tagged” or “untagged” Ethernet frames.

The EdgeMarc supports tagged and untagged VLANs. As specified in the IEEE 802.1q standard, tagged VLANs incorporate the VLAN ID and priority in the packet header. Untagged VLAN packets do not include the VLAN IDor priority. Most EdgeMarc models provide support for multiple tagged VLANs. All EdgeMarc models support a maximum of 16 VLANs configured at one time. The following VLAN IDs were used on the EdgeMarc by default:

- VLAN ID 1 (formerly 2730)—Interface management

- VLAN ID 500—Voice

- VLAN ID 600—Data

- To begin configuring the LAN side VLAN, click on the link, “VLAN Configuration” in the LAN settings. Note that the EdgeMarc supports LAN side VLAN and WAN side VLAN.

Lkjhlkhqwlerfqwer

- Create a new VLAN ID = 500. Enter the IP Address & Subnet Mask of the EdgeMarc for the associated Subnet.  We will not add IPv6 addressing in the lab, but note that EdgeMarc does support IPv6 addressing for LAN side VLAN.  The virtual IP address fields remain blank. They are associated with the implementation of EdgeMarc “High Availability” functionality.

- The LAN VLAN configuration page should now include two VLANs.

- Select VLAN Port and configure Port 7 and 8 to accept untagged packets with PVIP = 500.

- Configure EdgeMarc’s built in VoIP ALG (Application Layer Gateway) to work with the VoIP specific VLAN. Enable VLAN Support.

- Click on the link to the VoIP ALG – Application Layer Gateway configuration page. The EdgeMarc built in ALG must know which VLAN ID is going to be associated with VoIP traffic so that it can provide support to VoIP traffic passing through the EdgeMarc.

- Select the VLAN ID that will be associated with all VoIP devices on the LAN side of the EdgeMarc.

Ensure the EdgeMarc VoIP ALG is configured to be used with VLAN ID = 500.

Part 2: WAN VLAN Configuration

When setting up WAN VLAN it is important to ensure that VLAN support is configured in the WAN side Ethernet switch.

- Use the WAN VLAN Configuration page to configure the system WAN Ethernet port to support tagged VLANs. By default, all traffic (including data and voice) is tagged with VLAN ID 1.

- Note the warning message stating that “WAN VLAN is not currently enabled on the “Network” Page. Use the link provided to return to the main network page and enable IPv4 WAN VLAN.

- Select the WAN IPv4 VLAN enable button. Note that a link back the WAN VLAN configuration page appears.

# Questions

- Why is VLAN popular in VoIP infrastructure deployment architectures?

- Does the EdgeMarc support “tagged” or “untagged” LAN site VLAN Ethernet frames? Explain.

- Why does the EdgeMarc support VLAN on the WAN side?

- In one sentence (or less!) explain what the EdgeMarc VoIP ALG “does”?

- What difference does it make if the EdgeMarc’s built in VoIP ALG is unaware of which VLAN is being used for all VoIP equipment?