# Lab 3.5: Setting up DHCP Server and DHCP Relay Agent

# Lab Overview

EdgeMarc has three possible configuration options associated with DHCP:

- DHCP Server

- DHCP Relay Agent

- Disable all DHCP service

The EdgeMarc 4700 default configuration includes an enabled DHCP server. The EdgeMarc internal LAN interface is assigned IPv4 address 192.168.1.1/24. Connecting DHCP clients receive addresses starting with 192.168.1.100/24. The EdgeMarc DHCP server can be configured to assign addresses from any range. The built in DHCP server includes the ability to configure all the key DHCP “Options”.

Options are used to inform connecting clients about associated network services such as the IP address or FQDN (Fully Qualified Domain Name) of important support servers. For example, when setting up the EdgeMarc DHCP server, the address of an FTP (File Transfer Protocol) or TFTP (Trivial FTP) Server can be included. EdgeMarc DHCP server can also inform clients of the address / FQDN of a Network Time Server. EdgeMarc also supports the very useful “Option 129” VLAN Discovery. When a VoIP device boots up in its default configuration, the EdgeMarc DHCP server can inform it of the VLAN ID associated with the VoIP VLAN.

Operating as a DHCP Relay Agent, the EdgeMarc can provide an pathway for LAN devices to connect to a centralized WAN DHCP Server.

# Duration

- 30 minutes

# Prerequisites

None

# Objectives

- After successful completion of this lab, students will be able to:

  - Enable the EdgeMarc DHCP Server or DHCP Relay Agent

  - Understand why the EdgeMarc is a powerful, feature rich DHCP Server for local, LAN based VoIP equipment.

# Lab Equipment – Per Group Requirement

- Computer running Wireshark

- EM 4700

# Shared Equipment – Demos & Labs

- Lab network

# Supporting Documents

EM User Guides

# Procedure

Part 1: Enable and Configure EdgeMarc DHCP Server

# Questions

- Explain the difference between the EdgeMarc functioning as a DHCP Server vs a DHCP Relay agent.

- Why does the EdgeMarc DHCP server provide the ability to include a number of “options”?  Are these options part of the IETF standards for DHCP?

- What are the primary differences between FTP server and TFTP server?

- Map the message flow for DHCP between a client device and the EdgeMarc.